Objective:

Flash Software test via VFLASHPACK and CAPL

SW Tool Required:

Canoe Pro + VFLASH license

HW Required:

ECU + Vector box + Power supply

Procedure:

- Create Test Environment in Test Module

- Right-click on Test Setup and Create New Test Environment

- Add the CAPL Test module under the new Test Environment.

- Right-click on the added “CAPL Test Module”. Click on “Configuration”

- Update Name and add Can file in the Test Script

- Go to the components -> Add vFlashNodeLayer.dll/ vFlashNodeLayer.vmodule. This file could be found under the installation folder of Canoe inside Exec64 folder as shown below:

- Click on “OK” to save the configuration.

- Right-click on the CAPL test module. Click on the “Edit” button. The CAPL browser will be open.

- Vflash_main.can will open as shown below:

- Update Vflash_main.can file based on the test scenarios. Click on the Compile button for compilation issues. If no errors, you can start the Canoe simulation.

- Go to the CAPL Test module “Vflash_Main”. Right-click on it and select “Execution”.

- New window will open. Click on the yellow highlighted button to execute and red button to see the executed report.

Test scenarios available in CAN file as of now:

- Single-time flash test with flashing time verification

- Multiple flash Test with flashing time verification

- Signature, Checksum corruption test

- Diagnostic services applicable in Appl/bootloader -> Function template is available. You can use it for all the services.